Supporting Information

Molecular Mechanisms of Adsorbing Triclocarban by the Activated Sludge in Wastewater Treatment Systems

Hao Wanga, Hanlin Cuia, Xiaodan Mab, Carolyn R. Cornellc, Liying Zhanga, Yijun Renb, Minghan Lia, Ying Liua, Shu-Hong Gaob, Zhiling Lia, Ai-Jie Wanga,b, Jizhong Zhoud,e, Bin Liangb*

aState Key Laboratory of Urban Water Resource and Environment, School of Environment, Harbin Institute of Technology, Harbin 150090, China

bState Key Laboratory of Urban Water Resource and Environment, School of Civil & Environmental Engineering, Harbin Institute of Technology Shenzhen, Shenzhen 518055, China

cDepartment of Civil and Environmental Engineering, Rice University, Houston, Texas,

9 77005, USA

dInstitute for Environmental Genomics and Department of Microbiology and Plant Biology, University of Oklahoma, Norman, OK 73019, USA

eSchool of Civil Engineering and Environmental Sciences, University of Oklahoma, Norman, OK 73019, USA

*Corresponding author. State Key Laboratory of Urban Water Resource and Environment, School of Civil & Environmental Engineering, Harbin Institute of Technology Shenzhen, Shenzhen, 518055, China. E-mail: liangbin1214@hit.edu.cn (B. Liang).

### Experimental device and operation

The HA-A/O reactor (Fig. 1a), made of polymethyl methacrylate with a working volume of 5.0 L, was employed. The volumetric ratio of anaerobic (AN), anoxic (ANO) and oxic (O) sections is 6:1:3, and the secondary sedimentation tank was vertical type flow cell with a working volume of 0.95 L. The raw sludge was collected from the secondary sedimentation tank of Taiping Wastewater Treatment Plant (Harbin, China) and inoculated into the HA-A/O system. Agitators were installed in the anoxic and oxide zones, and aeration tubes were installed in the oxide zone to ensure that the dissolved oxygen was 3-5 mg/L and that the activated sludge was evenly distributed. The mixed liquor suspended solids (MLSS) concentration in anaerobic, anoxic and oxic section is 4300 mg/L, 3000 mg/L and 3000 mg/L, respectively. The hydraulic retention time (HRT) is 24 h, and the sludge retention time (SRT) is 15 d. The HA-A/O reactor was fed with a synthetic wastewater (Tabel S1) representative of domestic wastewater in China proposed elsewhere ensuring no background antimicrobial agents occurrence.

### Chemical structures and properties of TCC

Triclocarban (TCC)

CAS No: 101-20-2

Formula: C13H9Cl3N2O

Molecular Weight: 315.58 g mol-1

### EPSs extraction and analysis

The EPS, a dynamic two-layer membrane structure, includes a loosely and tightly bound layer, i.e., LB-EPSs and TB-EPSs. The methods for EPSs extraction and evaluation were referenced with previous studies[1]. The specific method was listed as follows: Firstly, 10 mL sludge sample (from An, Ano, and O section) was centrifuged at 4000 g for 10 min, and the filtrate obtained by 0.45 μm cellulose nitrate membrane filter was considered as S-EPS. Secondly, the volume of residue was adjusted to 10 mL by 0.05% NaCl with temperature of 70 °C, and the mixture was mixing quickly by a vortex, the filtrate was served as LB-EPSs. Thirdly, after LB-EPS extraction, the residue in centrifuge tube was adjusted to original volume by 0.05% NaCl with temperature of 70 °C, and the mixture was mixing quickly by a vortex, then the centrifuge tube was treated for 30 min at a water-bath shaker with temperature of 60 °C, finally the filtrate was considered as TB-EPSs. As PSs and PNs were the two main components in EPSs, the changes of them were employed to evaluate the effects of TCC on EPSs.

Soluble PSs were measured by phenol‑sulfuric acid method, and soluble PNs were detected by a protein kit (Sangon Biotech Co., Ltd., Shanghai).

### Removal efficiency and specific adsorption of TCC in batch experiments

The removal efficiency and the specific adsorption (qt) of TCC was calculated according to the following equations (1) and (2):

Removal efficiency (%) = (c0-ct)c0 × 100%                                   (1)

qt= c0-ctVm                                                             (2)

Where qt (μg/g) is the amount of TCC in the absorbent at time t (hr); c0 (μg/L) is the initial TCC concentration; ct (μg/L) is the TCC concentration in aqueous phase at time t (hr); V (L) is the solution volume; m (g) is the mass of adsorbent.

Adsorption isotherms

The adsorption capacity of TCC onto AN, Ano and O sludge was measured. The experimental data were fitted with the Freundlich (3) and the Langmuir (4) isotherms as shown below.

qe = Kf ce1/N                                                         (3)

qe= bqmce1+bce                                                           (4)

Where Kf is the binding energy constant reflecting the affinity of adsorbents to TCC; N is the Freundlich nonlinearity index; ce (μg/L) is the equilibrium concentration of TCC in aqueous phase; qe (μg/g) is the amount of CIP in adsorbent at equilibrium;

qm (μg/g) is the theoretical maximum adsorption capacity; b (L/μg) is the Langmuir constant related to adsorption energy.

### Bacterial community composition analysis of metagenomic sequencing data

The sludge microbial communities from different reactor sections were collected and centrifuged at 12,000 rpm for 15 min. The DNA sequencing were performed by Majorbio Bio-Pharm Technology Co., Ltd, Shanghai, China. For the metagenomics analysis of the samples of day 68, 69, and 70. raw sequencing reads from the HiSeq platform were filtered using Trimmomatic (v0.36) software to remove low quality (score < 20) and ambiguous N bases[2]. The paired-reads were merged using the shuffleSequences_fastq.pl Perl script[2]. High-quality metagenomic reads were assembled into contigs using the de novo assembler of IDBA-UD software (v1.1.2) applying a kmer of 20–100[3]. Only contigs longer than 500 bp were further analyzed, and the quality of contigs was evaluated using Quast (v4.6.3)[4]. Taxonomic information of the obtained contigs was validated using CAT (v2.0) software based on RefSeq database[5].

Table S1. The composition of the synthetic wastewater

| Components | Concentration(/L) |
| --- | --- |
| NH4Cl | 0.0764g |
| KNO3 | 0.036 g |
| KH2PO4 | 0.0136 g |
| NaHCO3 | 0.1 g |
| CaCl2 | 0.004 g |
| CH3COONa | 0.146 g |
| C3H6O | 0.26 mL |
| Trace element solution | 1 mL |
| Vitamins solution | 1 mL |

Table S2. Constants and correlation coefficients of three adsorption isotherms for TCC adsorption by (Sludge+EPS), Sludge (after extracting EPS), and EPS in HA-A/O systems.

| Section | Adsorption isotherms | Parameters | Sludge + EPS | EPS | Sludge |
| --- | --- | --- | --- | --- | --- |
| AN | Langmuir | qm (μg/g) | 0.7495±0.0170 | 7.9735±0.407 | 0.6061±0.0115 |
|  |  | k (L/μg) | 0.0223±0.0066 | 0.0147±0.0130 | 0.0243±0.0057 |
|  |  | R2 | 0.9726 | 0.8860 | 0.9831 |
|  | Freundlich | Kf | 0.7030±0.004 | 7.6061±0.2433 | 0.5661±0.0034 |
|  |  | N | 0.0482±0.003 | 0.0512±0.0176 | 0.0475±0.0033 |
|  |  | R2 | 0.9977 | 0.9320 | 0.9974 |
| ANO | Langmuir | qm (μg/g) | 0.8940±0.0184 | 6.7417±0.192 | 0.6163±0.018 |
|  |  | k (L/μg) | 0.0756±0.0102 | 0.0157±0.007 | 0.1082±0.016 |
|  |  | R2 | 0.9839 | 0.9615 | 0.9541 |
|  | Freundlich | Kf | 0.7503±0.0187 | 6.3794±0.1343 | 0.6177±0.0189 |
|  |  | N | 0.0950±0.0132 | 0.0334±0.0116 | 0.1078±0.0161 |
|  |  | R2 | 0.9661 | 0.9682 | 0.9543 |
| O | Langmuir | qm (μg/g) | 0.8503±0.0268 | 6.8593±0.1859 | 0.7288±0.0251 |
|  |  | k (L/μg) | 0.0503±0.025 | 0.0089±0.0071 | 0.0636±0.0155 |
|  |  | R2 | 0.9594 | 0.9871 | 0.9523 |
|  | Freundlich | Kf | 0.5664±0.0037 | 6.6760±0.1311 | 0.6200±0.0069 |
|  |  | N | 0.0472±0.0031 | 0.0233±0.0112 | 0.093±0.0059 |
|  |  | R2 | 0.9974 | 0.9708 | 0.9931 |

Table S3. Binding constants (Kb) and the number of binding sites (n) in EPS for TCC.

| Peak | Kb | n | R2 |
| --- | --- | --- | --- |
| EX305/EM435 | 0.42 | 0.54 | 0.9842 |
| EX310/EM370 | 96.22 | 0.3742 | 0.9658 |
| EX350/EM435 | 16.71 | 0.3675 | 0.9602 |
| EX280/EM340 | 132.95 | 0.4693 | 0.9472 |

| 68_32_k97_1189989_1_1 | 70_32_k97_213410_1_1 | Query | Table S3 Gene of AMT protein homology comparison between sludge samples of HA-A/O process |
| --- | --- | --- | --- |
| 11 | 11 | Subject |  |
| 72.2 | 75.6 | Identity (%) |  |
| 169 | 119 | Align length |  |
| 47 | 29 | Mismatches |  |
| 0 | 0 | Gaps |  |
| 1 | 2 | Query start |  |
| 169 | 120 | Query end |  |
| 58 | 158 | Subject start |  |
| 226 | 276 | Subject end |  |
| 1.06E-90 | 2.12E-62 | E-value |  |
| 258 | 183 | Bit score |  |

Table S4. Molecular dynamics simulation parameters

| Production | Equilibration | Equilibration | Heat | Minimization | Minimization | Minimization | Step |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 60 ns | 1 ns | 0.2 ns | 0.02 ns | 20000 steps | 5000 steps | 5000 steps | Number of steps or duration |
| NVT | NVT | NPT | NVT |  |  |  | System synthesis |
| Berendsen | Berendsen | Langevin | Langevin |  |  |  | Thermostat |
| 300 K | 300 K | 300 K | 300 K |  |  |  | Temperature |
|  | position constraint: mask:1-405@CA, C, N | position constraint: mask:1-406@H= |  |  | position constraint: mask:1-405@CA, C, N | position constraint: mask:1-406@H= | Constraint |

Fig. S1. Schematic diagram of the lab-scale Hydrolytic anaerobic-anoxic/oxic (HA-A/O) process

Fig. S2. FTIR spectra of S-EPS, LB-EPS and TB-EPS extracted from AN(abc),, Ano(def), and O (ghi) sludge samples in Stage 1 (Day 10, 0 mg L-1 of TCC, blue line) and Stage 4 (Day 60, 8 mg L-1 of TCC, red line).

### References

[1] Z.-W. He, W.-Z. Liu, C.-C. Tang, B. Liang, A.-J. Zhou, F. Chen, Y.-X. Ren, A.-J. Wang, Responses of anaerobic digestion of waste activated sludge to long-term stress of benzalkonium chlorides: Insights to extracellular polymeric substances and microbial communities, Sci Total Environ 796 (2021) 148957.

[2] A.M. Bolger, M. Lohse, B. Usadel, Trimmomatic: a flexible trimmer for Illumina sequence data, Bioinformatics 30 (2014) 2114-2120.

[3] Y. Peng, H.C.M. Leung, S.M. Yiu, F.Y.L. Chin, IDBA-UD: a de novo assembler for single-cell and metagenomic sequencing data with highly uneven depth, Bioinformatics 28 (2012) 1420-1428.

[4] A. Gurevich, V. Saveliev, N. Vyahhi, G. Tesler, QUAST: quality assessment tool for genome assemblies, Bioinformatics 29 (2013) 1072-1075.

[5] D.D. Cambuy, F.H. Coutinho, B.E. Dutilh, Contig annotation tool CAT robustly classifies assembled metagenomic contigs and long sequences, bioRxiv (2016) 072868.